Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

October 18, 2021

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Aerotech J615ST-20A RMS motor was tested on October 16, 2021 and is in compliance with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	Aerotech Division, RCS Rocket Motor Components

Manufacturer’s Designation	J615ST-20A      RMS [uses the 54/852 hardware]

Propellant	Super Thunder

TMT Nomenclature	745 J615

Diameter	2.125”	53.98 mm

Overall Length	9.466”	240.43 mm

Length Inside	9.094”	230.98 mm

Length Outside	0.372”	9.45 mm

Loaded Mass	1.768 lb	801.74 g

Propellant Mass	0.817 lb*	370.5 g*

Burnout Mass	0.896 lb	406.55 g

* data provided by manufacturer

Burn time	1.199 ± 0.046 sec

Total Impulse	167.58 ± 3.19 lbf.s	745.45 ± 14.20 N.s

Max Impulse	209.66 ± 27.77 lbf	933.94 ± 123.52 N

Average Impulse	138.27 ± 5.41 lbf	615.06 ± 24.08 N

Number of Motors Tested	6

Nominal	Measured

Delay Time Data	20	20.035, 20.094 sec

16	17.405, 17.438 sec

12	12.749, 12.653 sec

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing