Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

May 9, 2022

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The redesigned Aerotech M6000ST-P motor was tested on May 7, 2022 and is in compliance with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	Aerotech Division, RCS Rocket Motor Components

Manufacturer’s Designation	M6000ST-P                     [98/10240 hardware]

Propellant	Super Thunder

TMT Nomenclature	8247 M5959

Diameter	3.870”	98.3 mm

Overall Length	31.813”	808.04 mm

Length Inside	29.5”	749.3 mm

Length Outside	2.3125”	58.74 mm

Loaded Mass	17.7 lb	8028.58 g

Propellant Mass	8.527 lb*	3868 g*

Burnout Mass	9.02 lb	4091.4 g

* data provided by manufacturer

Burn time	1.382 sec

Total Impulse	1854 lbf.s	8247.1 N.s

Max Impulse	1526.2 lbf	6788.7 N

Average Impulse	1339.6 lbf	5958.9 N

Number of Motors Tested	3

Nominal	Measured

Delay Time Data	n.a.	n.a.

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing