Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

May 9, 2022

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Aerotech K750ST-P motor was tested on May 7, 2022 and is in compliance with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	Aerotech Division, RCS Rocket Motor Components

Manufacturer’s Designation	K750ST-P                     [75/1280 hardware]

Propellant	Super Thunder

TMT Nomenclature	1299 K747

Diameter	2.971”	75.46 mm

Overall Length	10.5”	266.7 mm

Length Inside	9.742”	247.5 mm

Length Outside	0.758”	19.25 mm

Loaded Mass	4.2934 lb	1947.5 g

Propellant Mass	1.312 lb*	595g*

Burnout Mass	1.7185 lb	779.52 g

* data provided by manufacturer

Burn time	1.740 ± .004 sec

Total Impulse	292.42 ± 0.83 lbf.s	1298.73 ± 6.40 N.s

Max Impulse	187.87 ± 3.55 lbf	835.69 ± 15.79 N

Average Impulse	167.83 ± 0.51 lbf	746.56 ± 2.26 N

Number of Motors Tested	3

Nominal	Measured

Delay Time Data	n.a.	n.a.

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing