Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

October 31, 2022

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Classic Enerjet by Aerotech F67C single-use motor was tested on October 26, 2022 and complies with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	RCS Rocket Motor Components, Inc.

Manufacturer’s Designation	{F67-6,9,14C}       [single use]

Propellant	Classic

TMT Nomenclature	77 F55

Diameter	1.131”	28.74 mm

Overall Length	4.42”	112.35 mm

Outside the MMT	0.94”	23.97 mm

Loaded Mass	0.189 lb	85.67 g

Propellant Mass	0.0811 lb*	36.8 g*

Burnout Mass	0.096 lb	43.38 g

* = data provided by manufacturer

Burn time	1.411 ± 0.057 sec

Total Impulse	17.34 ± 0.254 lbf.s	77.46 ± 1.867 N.s

Max Impulse	16.363 ± 1.092 lbf	72.79 ± 4.859 N

Average Impulse	12.413 ± 0.342 lbf	55.22 ± 1.519 N

Number of Motors Tested	10

Nominal	Measured

Delay Time Data	6 s	7.072, 6.62, 6.052, 5.938 s

9 s	9.078, 8.723, 9.115 s

14 s	12.883, 12.526, 12.766 s

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing