Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

December 6, 2022

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Aerotech K62N-P single-use motor was tested on December 4, 2022 and complies with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	RCS Rocket Motor Components, Inc.

Manufacturer’s Designation	K62N-P       [single use, DMS]

Propellant	Warp Nine

TMT Nomenclature	1438 K63

Diameter	2.125”	53.98 mm

Overall Length	14.734”	374.25 mm

Outside the MMT	0.995”†	25.27 mm†

† = an estimate: tested motors were supplied without external thrust collar

Loaded Mass	2.815 lb	1276.9 g

Propellant Mass	1.832 lb*	831 g*

Burnout Mass	0.820 lb	372.1 g

* = data provided by manufacturer

Burn time	23.1 ± 1.40 sec

Total Impulse	323.4 ± 3.85 lbf.s	1438.5 ± 17.08 N.s

Max Impulse	58.2 ± 3.09 lbf	258.7 ± 13.75 N

Average Impulse	14.10 ± 1.04 lbf	62.54 ± 4.64 N

Number of Motors Tested	3

Nominal	Measured

Delay Time Data	n.a.	-

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing