Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

December 6, 2022

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Aerotech J1265ST-14A single-use motor was tested on December 4, 2022 and complies with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	RCS Rocket Motor Components, Inc.

Manufacturer’s Designation	J1265ST-14A       [single use, DMS]

Propellant	Super Thunder

TMT Nomenclature	1072 J1261

Diameter	2.125”	53.98 mm

Overall Length	15.641”	397.27 mm

Outside the MMT	1.375”	34.93 mm

Loaded Mass	2.414 lb	1095.1 g

Propellant Mass	1.1177 lb*	507 g*

Burnout Mass	1.212 lb	549.71 g

* = data provided by manufacturer

Burn time	0.85 ± 0.007 sec

Total Impulse	251.07 ± 17.04 lbf.s	1072.3 ± 1.563 N.s

Max Impulse	392.36 ± 57.25 lbf	1745.3 ± 254.7 N

Average Impulse	283.51 ± 1.542 lbf	1261.1 ± 6.9 N

Number of Motors Tested	3

Nominal	Measured

Delay Time Data	6 s	7.345 s

10 s	10.35 s

14 s	12.925 s

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing