Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

January 10, 2023

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Aerotech O5500X-PS single-use reload kit (RMS) was tested on January 7, 2023 and complies with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	RCS Rocket Motor Components, Inc.

Manufacturer’s Designation	O5500X-PS       [single use - DMS]

Propellant	Propellant X

TMT Nomenclature	21,384 O5515

Diameter	3.867”	98.45 mm

Overall Length	59.344”	1507.3 mm

Outside the MMT	2.438”	61.91 mm

Loaded Mass	37.00 lb	16,783 g

Propellant Mass	21.559 lb*	9779 g*

Burnout Mass	14.925 lb	6769.9 g

* = data provided by manufacturer

Burn time	4.026 sec

Total Impulse	4807 lbf.s	21,384 N.s

Max Impulse	1697.7 lbf	7551.5 N

Average Impulse	1239.7 lbf	5514.6 N

Number of Motors Tested	2

Nominal	Measured

Delay Time Data	n.a.	n.a.

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing