Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

April 30, 2023

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Aerotech N4000W-PS motor was tested on April 29, 2023 and is in compliance with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	Aerotech Division, RCS Rocket Motor Components

Manufacturer’s Designation	N4000W-PS                     [RMS, 98/20480 case]

Propellant	White Lightning

TMT Nomenclature	18,184 N3906

Diameter	3.878”	98.49 mm

Overall Length	56.406”	1432.7 mm

Length Inside	53.969”	1370.8 mm

Length Outside	2.438”	61.91 mm

Loaded Mass	36.10 lb	16,374.7 g

Propellant Mass	21.605 lb*	9,800 g*

Burnout Mass	14.165 lb	6,425.1 g

* data provided by manufacturer

Burn time	4.747, 4.563 sec

Total Impulse	4087.9 lbf.s	18,183.8 N.s

Max Impulse	1213.6 lbf	5398.3 N

Average Impulse	878.11 lbf	3905.9 N

Number of Motors Tested	2

Nominal	Measured

Delay Time Data	n.a.	n.a.

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing