Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

May 6, 2023

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Aerotech H14ST-P motor was tested on May 6, 2023 and is in compliance with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	Aerotech Division, RCS Rocket Motor Components

Manufacturer’s Designation	H14ST-P                     [DMS, single use]

Propellant	Super Thunder

TMT Nomenclature	240 H13

Diameter	1.129”	28.66 mm

Overall Length	9.438”	239.71 mm

Length Inside	8.755”	222.38 mm

Length Outside	0.682”	17.33 mm

Loaded Mass	0.506 lb	229.47 g

Propellant Mass	0.296 lb*	134.1 g*

Burnout Mass	0.168 lb	76.06 g

* data provided by manufacturer

Burn time	18.255 ± 0.36 sec

Total Impulse	53.982 ± 1.37 lbf.s	240.12 ± 6.09 N.s

Max Impulse	9.352 ± 0.984 lbf	42.13 ± 5.22 N

Average Impulse	2.971 ± 0.117 lbf	13.21 ± 0.52 N

Number of Motors Tested	3

Nominal	Measured

Delay Time Data	n.a.	n.a.

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing