[image: image1.png]HRR@L
ROCKETRY ASSOCIATION, INC. |




Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

August 20, 2018
Gary Rosenfield
RCS Rocket Motor Components, Inc.
2113 West 850 North Street
Cedar City, UT   84721
Dear Mr. Rosenfield,


The Aerotech J520W-13A RMS motor was tested on April 21, 2017, May 3, 2018, and August 14, 2018 and is in compliance with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer
Aerotech Division, RCS Rocket Motor Components
Manufacturer’s Designation
J520W-13A      RMS for the 38/1080 hardware

Propellant
White Lightning
TMT Nomenclature
671 J487
Diameter
-
38 mm
Overall Length
19.77”
502.12 mm
Length Inside
19.33”
490.93 mm

Length Outside
0.433”
11.19 mm

Loaded Mass
1.670 lb
757.42 g
Propellant Mass
0.840 lb*
381 g*
Burnout Mass
0.771 lb
349.65 g

         * data provided by manufacturer
Burn time
1.377 ± 0.026 sec
Total Impulse
150.9 ± 4.97 lbf.s
671.26 ± 22.09 N.s
Max Impulse
202.93 ± 23.49 lbf
902.66 ± 104.5 N
Average Impulse
109.57 ± 4.63 lbf
487.41 ± 20.60 N
Number of Motors Tested
6

Nominal                      Measured

Delay Time Data
5                                  6.227, 6.639 sec

9                                  9.332, 10.204 sec

13                                10.486, 10.047 sec                                                                                                                     12.989,12.531 sec

Casing Temp. [NFPA 1125 7.4.1]



Passed
[image: image2.jpg]Thrust (Ibf)

Aerotech J520W-13A

250

Time (sec)

1000

800

600

400

200

Thrust (N)




Sincerely,
Alan C. Whitmore

Chair, Tripoli Motor Testing
